RTL CHALLENGE

DAY – 25 :- Verilog Code for Implementation of  Gray to Binary Converter.

Software used:-  Xilinx  Vivado(2023.1)

Theory:-

A Gray to Binary converter is a digital circuit that converts Gray code, a binary numeral system where two successive values differ in only one bit, back into standard binary numbers. Here's an explanation of how a Gray to Binary converter works:

1. Inputs: The input to the converter is the Gray code that needs to be converted back to binary.

2. Conversion Algorithm: Converting Gray code to binary involves a slightly more complex algorithm compared to the binary-to-Gray  conversion. The algorithm typically involves a series of XOR operations as well.

3. Process: To convert Gray code back to binary:

 Start from the most significant bit (MSB) of the Gray code.

 Copy the MSB of the Gray code to the corresponding bit of the binary output.

 For each subsequent bit:

 Perform an XOR operation between the bit of the Gray code and the previously obtained bit of the binary output.

 The result becomes the next bit of the binary output.

4. Outputs: The output of the converter is the corresponding binary representation of the input Gray code.

5. Example: Let's convert the Gray code 1101 back to binary:

 Gray: 1 1 0 1

 Binary: 1 (1 XOR 1) (0 XOR 1) (1 XOR 0) = 1101

6. Implementation: Similar to the Binary to Gray converter, you can implement the Gray to Binary conversion algorithm using XOR gates in hardware or by writing a program in software.
7. Expansion: The conversion process can be scaled up for larger Gray code numbers by applying XOR operations to each pair of corresponding bits.

Code:-